Network Design and Configuration

(By- Surbhi Gupta)

Objectives:

• To plan the IP address assignments for the Enterprise internal network using efficient subnetting.

• To plan static routing to provide end-to-end connectivity in the network.

• To provide default route to connect internal devices to internet.

Scenario: UnoSecurity is an organization that deals with providing an access control system for hospitals, banks, and government buildings. The networking team had come up with the base topology and network plan for the network as per the new requirement shown

# Table of Contents

Part A ……………………………………………………………………………………………..3

Part B………………………………………………………………………………………………4

Part C………………………………………………………………………………………………8

Part D………………………………………………………………………………………………11

Tables and Figures……………………………………………………………………………….21

# Assignment

UnoSecurity is an organization that deals with providing an access control system for hospitals, banks, and government buildings. The networking team had come up with the base topology and network plan for the network

## Part A

According to the network plan given, the schematic is as below:

Figure 1: Router Interfaces with assigned IP addresses

## Part B

The static route configuration on the routers according to the constraints:

### R1 (Router 1)

| Label | Destination network address | Subnet Mask (dotted decimal) | Next hop IP address | Exit interface |
| --- | --- | --- | --- | --- |
| C | 172.16.0.0 | 255.255.255.0 | -------- | Gig 8/0 |
| C | 172.16.1.0 | 255.255.255.0 | -------- | Gig 9/0 |
| C | 172.16.255.0 | 255.255.255.252 | -------- | Gig 6/0 |
| C | 172.16.255.4 | 255.255.255.252 | -------- | Gig 7/0 |
| S | 172.16.2.0 | 255.255.255.224 | 172.16.255.2 | Gig 6/0 |
| S | 172.16.2.32 | 255.255.255.224 | 172.16.255.2 | Gig 6/0 |
| S | 172.16.255.8 | 255.255.255.252 | 172.16.255.2 | Gig 6/0 |
| S | 142.100.0.12 | 255.255.255.252 | 172.16.255.6 | Gig 7/0 |
| S* | 0.0.0.0 | 0.0.0.0 | 172.16.255.6 | Gig 7/0 |

Table 1: Routing Configuration for Router 1

Configuration Mode

ip route 172.16.2.0 255.255.255.192 172.16.255.2 {This is a summarized route}

ip route 172.16.255.8 255.255.255.252 172.16.255.2

ip route 142.100.0.12 255.255.255.252 172.16.255.6

ip route 0.0.0.0 0.0.0.0 172.16.255.6 {This is a default route}

Summarization

172.16.2.0/27 and 172.16.2.32/27 can be combined into a single summarized route.

In binary

| 172 | 16 | 2 | 0 |
| --- | --- | --- | --- |
| 10101100 | 0001000 | 00000010 | 00000000 |

| 172 | 16 | 2 | 32 |
| --- | --- | --- | --- |
| 10101100 | 0001000 | 00000010 | 00100000 |

There are 26 bits common in both, so the subnet mask should be /26 in CIDR notation.

In dotted decimal notation, the new subnet mask is 255.255.255.192

Therefore, R1 can summarize 172.16.2.0/27 and 172.16.2.32/27 as 172.16.2.0/26.

### R2 (Router 2)

| Label | Destination network address | Subnet Mask (dotted decimal) | Next hop IP address | Exit interface |
| --- | --- | --- | --- | --- |
| C | 172.16.2.0 | 255.255.255.224 | -------- | Gig 7/0 |
| C | 172.16.2.32 | 255.255.255.224 | -------- | Gig 8/0 |
| C | 172.16.255.0 | 255.255.255.252 | -------- | Gig 6/0 |
| C | 172.16.255.8 | 255.255.255.252 | -------- | Gig 9/0 |
| S | 172.16.0.0 | 255.255.255.0 | 172.16.255.1 | Gig 6/0 |
| S | 172.16.1.0 | 255.255.255.0 | 172.16.255.1 | Gig 6/0 |
| S | 172.16.255.4 | 255.255.255.252 | 172.16.255.10 | Gig 8/0 |
| S | 142.100.0.12 | 255.255.255.252 | 172.16.255.10 | Gig 8/0 |
| S* | 0.0.0.0 | 0.0.0.0 | 172.16.255.10 | Gig 8/0 |

Table 2: Routing Configuration for Router 2

Configuration Mode

ip route 172.16.0.0 255.255.254.0 172.16.255.1 {This is a summarized route}

ip route 172.16.255.4 255.255.255.252 172.16.255.10

ip route 142.100.0.12 255.255.255.252 172.16.255.10

ip route 0.0.0.0 0.0.0.0 172.16.255.10 {This is a default route}

Summarization

172.16.0.0 and 172.16.1.0 can be combined into a single summarized route.

In binary

| 172 | 16 | 0 | 0 |
| --- | --- | --- | --- |
| 10101100 | 0001000 | 00000000 | 00000000 |

| 172 | 16 | 1 | 0 |
| --- | --- | --- | --- |
| 10101100 | 0001000 | 00000001 | 00000000 |

There are 23 bits common in both, so the subnet mask should be /23 in CIDR notation.

In dotted decimal notation, the new subnet mask is 255.255.254.0

Therefore, R2 can summarize 172.16.0.0/24 and 172.16.1.0/24 as 172.16.0.0/23.

### R3 (Edge Router)

| Label | Destination network address | Subnet Mask (dotted decimal) | Next hop IP address | Exit interface |
| --- | --- | --- | --- | --- |
| C | 172.16.255.4 | 255.255.255.252 | -------- | Gig 8/0 |
| C | 172.16.255.8 | 255.255.255.252 | -------- | Gig 9/0 |
| C | 142.100.0.12 | 255.255.255.252 | -------- | Se 5/0 |
| S | 172.16.255.0 | 255.255.255.252 | 172.16.255.5 | Gig 8/0 |
| S | 172.16.0.0 | 255.255.255.0 | 172.16.255.5 | Gig 8/0 |
| S | 172.16.1.0 | 255.255.255.0 | 172.16.255.5 | Gig 8/0 |
| S | 172.16.2.0 | 255.255.255.224 | 172.16.255.9 | Gig 9/0 |
| S | 172.16.2.32 | 255.255.255.224 | 172.16.255.9 | Gig 9/0 |
| S* | 0.0.0.0 | 0.0.0.0 | 142.100.0.14 | Se 5/0 |

Table 3: Routing Configuration for Edge Router

Configuration Mode

ip route 172.16.255.0 255.255.255.252 172.16.255.5

ip route 172.16.0.0 255.255.254.0 172.16.255.5 {This is a summarized route}

ip route 172.16.2.0 255.255.255.192 172.16.255.9 {This is a summarized route}

ip route 0.0.0.0 0.0.0.0 142.100.0.14 {This is a default route}

Summarization

172.16.0.0 and 172.16.1.0 can be combined into a single summarized route.

In binary

| 172 | 16 | 0 | 0 |
| --- | --- | --- | --- |
| 10101100 | 0001000 | 00000000 | 00000000 |

| 172 | 16 | 1 | 0 |
| --- | --- | --- | --- |
| 10101100 | 0001000 | 00000001 | 00000000 |

There are 23 bits common in both, so the subnet mask should be /23 in CIDR notation.

In dotted decimal notation, the new subnet mask is 255.255.254.0

Therefore, R3 can summarize 172.16.0.0/24 and 172.16.1.0/24 as 172.16.0.0/23.

And now, 172.16.2.0/27 and 172.16.2.32/27 can also be combined into a single summarized route.

In binary

| 172 | 16 | 2 | 0 |
| --- | --- | --- | --- |
| 10101100 | 0001000 | 00000010 | 00000000 |

| 172 | 16 | 2 | 32 |
| --- | --- | --- | --- |
| 10101100 | 0001000 | 00000010 | 00100000 |

There are 26 bits common in both, so the subnet mask should be /26 in CIDR notation.

In dotted decimal notation, the new subnet mask is 255.255.255.192

Therefore, R3 can summarize 172.16.2.0/27 and 172.16.2.32/27 as 172.16.2.0/26.

### R4 (ISP Router)

| Label | Destination network address | Subnet Mask (dotted decimal) | Next hop IP address | Exit interface |
| --- | --- | --- | --- | --- |
| C | 142.100.0.12 | 255.255.255.252 | -------- | Se 2/0 |
| S* | 0.0.0.0 | 0.0.0.0 | 142.100.0.13 | Se 2/0 |

Table 4: Routing Configuration for ISP Router

Configuration Mode

ip route 0.0.0.0 0.0.0.0 142.100.0.13 {This is a default route}

## Part C

### 1. Syntax of Configuration Commands Required

Basic Router Configuration

enable			          # Enters privileged EXEC mode (requires the enable password or secret if configured).

config t			           # Enters global configuration mode, allowing changes to the router's settings.

hostname <hostname>             # Change router hostname (In our case, we have to set SG-R#)

enable secret cisco123              # Configures a secure, encrypted password for privileged EXEC mode.

line console 0	  # Enters console line configuration mode.

password cisco123                     # Sets the console password to 'cisco123'.

login		                          # Requires a password for console access.

exec-timeout 0 0	            # Disables automatic timeout for console sessions.

logging synchronous                  # Ensures system messages don’t interrupt command input

exit                                                 # Exits console line configuration mode.

Interface IP Address Assignment

interface <interface-name>                                 # Enters the specified interface configuration mode (e.g., GigabitEthernet0/0)

description <network description>	           # Adds a description to the interface for documentation purposes.

ip address <ip address> <subnet mask>            # Assigns an IP address and subnet mask to the interface.

no shutdown				 # Activates the interface (by default, interfaces are shut down).

Exit					 # Exits interface configuration mode.

Static Route Configuration

ip route <destination network> <mask> <next-hop_address or exitinterface> <administrative_distance> <permanent>

#Destination Network – Specifies the network we want to reach.

#Subnet Mask – Defines the network's subnet mask.

#Next-Hop Address or Exit Interface – Indicates the next-hop IP address or the router's outgoing interface.

#Administrative Distance – Sets the route's preference. Lower values mean higher priority.

#Permanent – Keeps the route in the routing table even if the next hop is unreachable.

DHCP Configuration

ip dhcp pool <pool_name>         # Creates a DHCP pool.

network <network address> <subnet mask>              # Defines network and subnet.

default-router <router_ip address>         # Sets default gateway for clients.

ip dhcp excluded-address <first ip address> <last ip address>         # Excludes IP range from DHCP assignment.

exit         # Exits pool configuration mode.

### 2. Syntax of Verification commands required

| Command | Purpose |
| --- | --- |
| For Routers |  |
| show ip interface brief | Verify the status of interfaces and ip address assignments to them. |
| show interface <interface> | Shows the additional interface information in addition to status, description and ip address which helps to monitor the specific interface properly. |
| show interfaces | Provides detailed IP settings, including IP address, subnet mask, ACLs, switching type, and compression settings of all interfaces. |
| show running-config | Checks the current configurations |
| show interfaces description | Lists all interfaces with their descriptions. |
| show ip route | Helps to view routing table of the router and verify static routes. |
| show dhcp binding | Displays DHCP-leased IP addresses |
| show ip dhcp pool | Displays details about the configured DHCP pool, including the number of leased and available addresses |
| show run ¦ include ip dhcp excluded-address | Verifies the IP addresses that have been excluded from the DHCP pool |
| For PCs |  |
| ping <destination ip address> | Tests the network connectivity |
| traceroute <destination ip address> | Checks the path taken by packets |
| ipconfig /all | Displays detailed network configuration information |

Table 5: Syntax for Verification Commands

Note: show interfaces description does not work on certain devices or some IOS versions. This command does not work with the router I used in the Cisco lab.

### 3. A Connectivity Check Table

| Source Network | Destination Network | Connectivity Check (Success/Fail) |
| --- | --- | --- |
| HR-Network | IT-Network | Success |
| HR-Network | Guest-Network | Success |
| HR-Network | Acc-Network | Success |
| Acc-Network | IT-Network | Success |
| Acc-Network | Guest-Network | Success |
| IT-Network | Guest Network | Success |
| R1 | R2 | Success |
| R1 | R3 | Success |
| R1 | R4 ISP Router | Success |
| R2 | R3 | Success |
| R2 | R4 ISP Router | Success |
| R3 | R4 ISP Router | Success |
| HR-Network | R4 ISP Router | Success |
| Acc-Network | R4 ISP Router | Success |
| IT-Network | R4 ISP Router | Success |
| Guest Network | R4 ISP Router | Success |

Table 6: A Connectivity Check Table

## Part D

Figure 2: Screenshot of Prototype created

!Configuration for Router 1

enable

configure terminal

!Configuring hostname

hostname SG-R#

!Setting password

enable password cisco123

!Basic router configuration

line console 0

password cisco123

login

exec-timeout 0 0

logging synchronous

exit

!Configuring Interfaces

interface Gig8/0

ip address 172.16.0.1 255.255.255.0

description Connection to HR-Network

no shutdown

exit

interface Gig9/0

ip address 172.16.1.1 255.255.255.0

description Connection to Acc-Network

no shutdown

exit

interface Gig7/0

ip address 172.16.255.5 255.255.255.252

description Link to Router 3 (R3)

no shutdown

exit

interface Gig6/0

ip address 172.16.255.1 255.255.255.252

description Link to Router 2 (R2)

no shutdown

exit

!Configuring Static Routes

ip route 172.16.2.0 255.255.255.192 172.16.255.2

ip route 172.16.255.8 255.255.255.252 172.16.255.2

ip route 142.100.0.12 255.255.255.252 172.16.255.6

ip route 0.0.0.0 0.0.0.0 172.16.255.6

!DHCP Configuration

ip dhcp pool HR_Pool

network 172.16.0.0 255.255.255.0

default-router 172.16.0.1

ip dhcp excluded-address 172.16.0.1 172.16.0.5

ip dhcp pool Acc_Pool

network 172.16.1.0 255.255.255.0

default-router 172.16.1.1

ip dhcp excluded-address 172.16.1.1 172.16.1.5

!Privileged EXEC-Mode

exit

!Viewing Routing table

show ip route

!Viewing dhcp configuration

show ip dhcp pool

!Configuration for Router 2

enable

configure terminal

!Configuring hostname

hostname SG-R#

!Setting password

enable password cisco123

!Basic router configuration

line console 0

password cisco123

login

exec-timeout 0 0

logging synchronous

exit

!Configuring Interfaces

interface Gig7/0

ip address 172.16.2.1 255.255.255.224

description Connection to IT-Network

no shutdown

exit

interface Gig8/0

ip address 172.16.2.33 255.255.255.224

description Connection to Guest-Network

no shutdown

exit

interface Gig6/0

ip address 172.16.255.2 255.255.255.252

description Link to Router 1 (R1)

no shutdown

exit

interface Gig9/0

ip address 172.16.255.9 255.255.255.252

description Link to Router 3 (R3)

no shutdown

exit

!Configuring Static Routes

ip route 172.16.0.0 255.255.254.0 172.16.255.1

ip route 172.16.255.4 255.255.255.252 172.16.255.10

ip route 142.100.0.12 255.255.255.252 172.16.255.10

ip route 0.0.0.0 0.0.0.0 172.16.255.10

!DHCP Configuration

ip dhcp pool IT_Pool

network 172.16.2.0

default-router 172.16.2.1

ip dhcp excluded-address 172.16.2.1 172.16.2.5

ip dhcp pool Guest_Pool

network 172.16.2.32

default-router 172.16.2.33

ip dhcp excluded-address 172.16.2.33 172.16.2.37

!Privileged EXEC-Mode

exit

!Viewing Routing table

show ip route

!Viewing dhcp configuration

show ip dhcp pool

!Configuration for Router 3

enable

configure terminal

!Configuring hostname

hostname SG-R#

!Setting password

enable password cisco123

!Basic router configuration

line console 0

password cisco123

login

exec-timeout 0 0

logging synchronous

exit

!Configuring Interfaces

interface Gig8/0

ip address 172.16.255.6 255.255.255.252

description Link to Router 1 (R1)

no shutdown

exit

interface Gig9/0

ip address 172.16.255.10 255.255.255.252

description Link to Router 2 (R2)

no shutdown

exit

interface Se5/0

ip address 142.100.0.13 255.255.255.252

description Link to ISP Router (R4)

no shutdown

exit

!Configuring Static Routes

ip route 172.16.255.0 255.255.255.252 172.16.255.5

ip route 172.16.0.0 255.255.254.0 172.16.255.5

ip route 172.16.2.0 255.255.255.192 172.16.255.9

ip route 0.0.0.0 0.0.0.0 142.100.0.14

!Privileged EXEC-Mode

exit

!Viewing Routing table

show ip route

!Configuration for ISP Router

enable

configure terminal

!Configuring hostname

hostname SG-R#

!Setting password

enable password cisco123

!Basic router configuration

line console 0

password cisco123

login

exec-timeout 0 0

logging synchronous

exit

!Configuring Interfaces

interface Se2/0

ip address 142.100.0.14 255.255.255.252

description Link to Router 3 (R3)

no shutdown

exit

!Configuring Static Routes

ip route 0.0.0.0 0.0.0.0 142.100.0.13

!Privileged EXEC-Mode

exit

!Viewing Routing table

show ip route

# Tables and Figures

Figure 1: Router Interfaces with assigned IP addresses	1

Figure 2: Screenshot of Prototype created	9

Table 1: Routing Configuration for Router 1	2

Table 2: Routing Configuration for Router 2	3

Table 3: Routing Configuration for Edge Router	4

Table 4: Routing Configuration for ISP Router	5

Table 5: Syntax for Verification Commands	8

Table 6: A Connectivity Check Table	8