Network Design and Configuration

(By- Surbhi Gupta)

Objectives:

To successfully configure Virtual LANs and inter-VLAN routing.

• To successfully configure EIGRP routing.

• To successfully configure packet filtering using Standard or Extended ACL [Bonus Question]

Scenario:

UnoSecurity specializes in providing access control systems for hospitals, banks, and government buildings. As the business is growing new branch office is added in the network. There is requirement of managing the network traffic and Network performance. The networking team at UnoSecurity want to improve the implementation design and restructure the network to improve for efficiency, network performance and availability. Following this article by Cisco Press, Hierarchical Network Design Overview (1.1) > Cisco Networking Academy Connecting Networks Companion Guide: Hierarchical Network Design | Cisco Press , they have decided to restructure the internal network at both the branches.

The team lead has requested you to create prototype for the proposed network

# Table of Contents

B

Bonus Question:	9

R

References	15

T

Tables and Figures	14

Task 1: Configure VLAN	3

Task 2: Configure intervlan Routing	6

Task 3: Configure Routing	8

V

Verification Commands	13

# Task 1: Configure VLAN

## VLAN Table

| VLAN Name | VLAN ID |
| --- | --- |
| Operations-Net | VLAN 10 |
| HR-Net | VLAN 20 |
| Guest-Net | VLAN 30 |
| Acc-Net | VLAN 40 |
| IT-Net | VLAN 50 |
| Mgmt-Net | VLAN 60 |

Table 1: VLAN Table

## For the UnoSecurity, Branch Office

| Switch | Connected To | Devices Connected | Final Role |
| --- | --- | --- | --- |
| Switch 5 | All switches + Router (BO) | None | Core Switch |
| Switch 4 | All access switches + Switch 5 | None | Aggregation Switch |
| Switch 0-1(1) | Switch 4 + Switch 5 | One PC each | Access Switches |

Table 2: Connections in Branch Office

Switch 5 – Core Switch

- It is directly connected to the main router and all other switches.

- It is at the top of the hierarchy and acts as the central bridge to route external traffic.

- It handles traffic between distribution/aggregation and external routers or the internet.

Final Role: Core Switch

Switch 4 – Distribution/Aggregation Switch

- It connects to all access switches and connects upstream to the core (Switch 5).

- Even though Switch 5 is also connected to access switches, Switch 4 aggregates local VLAN traffic from all access switches before possibly forwarding it to the core switch.

Final Role: Aggregation/Distribution Switch

Even if both Switch 4 and Switch 5 are connected to the access switches, Switch 4 can still serve as the aggregation/distribution point if it handles more east-west VLAN traffic within the branch LAN before it’s passed to the core (Switch 5) for routing.

Switches 0, 1, 0(1), 1(1) – Access Switches

- They connect directly to end devices (PCs) and have uplinks to both Switch 4 and Switch 5.

- These are classic access switches in a three-tier architecture.

Final Role: Access Layer

### Summary Table for VLAN Mapping

| Switch | Function | PC Attached | VLAN | Access Port | Trunk Ports | VLAN Name |
| --- | --- | --- | --- | --- | --- | --- |
| Switch 5 | Core | No | --- | --- | Fa0/1-2, Fa0/22-24 | --- |
| Switch 4 | Aggregation | No | --- | --- | Fa0/1, Fa0/21-24 | --- |
| Switch 0 | Access | PC0 | VLAN 10 | Fa0/24 | Fa0/22-23 | Operations-Net |
| Switch 1 | Access | PC1 | VLAN 20 | Fa0/24 | Fa0/22-23 | HR-Net |
| Switch 0(1) | Access | PC0(1) | VLAN 30 | Fa0/24 | Fa0/22-23 | Guest-Net |
| Switch 1(1) | Access | PC1(1) | VLAN 30 | Fa0/24 | Fa0/22-23 | Guest-Net |

Table 3: Summary Table for VLAN Mapping in Branch Office

Figure 1: Branch Office Configuration

## For the UnoSecurity, Head Office

| Switch | Connected To | Devices Connected | Final Role |
| --- | --- | --- | --- |
| Multi-Layered Switch 0 | All switches + Router (BO) | None | Core Switch |
| Multi-Layered Switch 1 | All access switches + Multi-Layered Switch 0 | None | Aggregation Switch |
| Switch 2 | Both Multi-Layered Switches | PC2 | Access Switch |
| Switch 3 | Both Multi-Layered Switches | Laptop0, Laptop1, Account Server, Web & DNS Server | Access Switch |

Table 4: Connections in Head Office

Similarly,

### Summary Table for VLAN Mapping

| Switch | Function | PC Attached | VLAN | Access Port | Trunk Ports | VLAN Name |
| --- | --- | --- | --- | --- | --- | --- |
| Multi-Layered Switch 0 | Core | No | --- | --- | Fa0/1, Fa0/22-24 | --- |
| Multi-Layered Switch 1 | Aggregation | No | --- | --- | Fa0/22-23 | --- |
| Switch 2 | Access | PC2 | VLAN 50 | Fa0/24 | Fa0/22-23 | IT-Net |
| Switch 3 | Access | Laptop0, Laptop1 | VLAN 50 | Fa0/23-24 | Fa0/20-21 | IT-Net |
| Switch 3 | Access | Account Server | VLAN 40 | Fa0/19 | Fa0/20-21 | Acc-Net |
| Switch 3 | Access | Web & DNS Server | VLAN 60 | Fa0/22 | Fa0/20-21 | Mgmt-Net |

Table 5: Summary Table for VLAN Mapping

Figure 2: Head Office Configuration

# Task 2: Configure intervlan Routing

## In Branch Office,

Figure 3: Router-on-a-stick has been configured

## Sub interfaces Configured

| Port | VLAN ID | IP Address |
| --- | --- | --- |
| FastEthernet0/1.10 | VLAN 10 | 192.168.2.1/24 |
| FastEthernet0/1.20 | VLAN 20 | 192.168.3.1/24 |
| FastEthernet0/1.30 | VLAN 30 | 192.168.0.1/23 |

Table 6: Sub interfaces Of Router

## In Head Office,

Figure 4: L3 Switch connecting to Router has been configured

## Ports Configured

| Port | VLAN ID | IP Address |
| --- | --- | --- |
| VLAN 40 | 40 | 172.16.1.1/24 |
| VLAN 50 | 50 | 172.16.2.1/24 |
| VLAN 60 | 60 | 172.16.3.1/24 |

Table 7: Ports Configured on Multilayered Switch 0

# Task 3: Configure Routing

## Configuring EIGRP

Figure 5: EIGRP Routing Protocol has been configured on both router and L3 Switch

## Configuring Static Route

Figure 6: Configuring Static Route between ISP and Router(HO)

# Bonus Question:

- Guest network should have access to web server only.

Here, the guest network is 192.168.0.0/23 and the server is 172.16.3.100. I am applying the access control list on the outer interface of Router(BO).

Figure 7: Accessing only web services through guest network

On PC(1), open web browser, Typed in ip address of web server. It will show this page. It means web server is reachable. Port 80 and port 443 is used for forming ACL list.

| Port | Protocol | Purpose |
| --- | --- | --- |
| 80 | HTTP | Unencrypted Web traffic |
| 443 | HTTPS | Encypted Web traffic |

Table 8: Ports and Functions

Figure 8: Guest-network cannot access any other device on the network

The devices in this network can only use web services but cannot access any other networks or even ping the web server. This means they can interact with the web services, but not communicate directly with the server or other networks.

- Only 192.168.3.10 host from the HR network can access the account server.

In this, I have again put ACL on Router(BO), so that only the host 192.168.3.10 from the HR network can access the account server 172.16.1.100, disabling the access to all other computers in HR Network.

For this, I have joined PC4 to vlan 20 temporarily to verify the ACL working.

- Pinging 172.16.1.100 from 192.168.3.10

Figure 9: pinging HR-host to Account Server

- Pinging 172.16.1.100 from the any other computer from HR Network

Figure 10: Destination host unreachable from other devices on HR-Network

- Only Account network can access the Account Server.

For this, I have put ACL on Multilayered Switch 0 allowing the host mentioned in part 2 i.e. 192.168.3.10 and all the devices in account network restricting any device from other networks. For checking, I have joined PC5 to vlan 40

- Pinging Account server 172.16.1.100 from PC5 172.16.1.10

Figure 11: Pinging Account server from device in Account network

- Pinging Account Server from device in Mgmt-Network

Figure 12: Pinging Account server from other networks

- Only IT network can access the DNS Server

Figure 13: Ping is successful from IT Network to DNS Server

# Verification Commands

| ipconfig | Show Ip configuration of a device |
| --- | --- |
| ping <ip> | Check connectivity |
| tracert <ip> | Traceroute path |
| show ip interface brief | Check IP and interface status |
| show interfaces | Show detailed interface info |
| show interfaces switchport | Show interface status and VLAN |
| show vlan brief | Show all VANs configured |
| show vlan | Check which interfaces are in which VLAN |
| show interfaces trunk | Show trunking interfaces |
| show ip route | Show ip routing table |
| show ip eigrp neighbors | Show EIGRP neighbors |
| show ip eigrp interfaces | Show EIGRP interfaces |
| show ip eigrp topology | Show EIGRP topology table |
| show access-lists | Show ACLs and their rules |
| show ip interfaces brief | Show SVI status |
| show ip protocols | Show routing protocols |
| show cdp neighbors | Show directly connected Cisco neighbors |

Table 9: Verification commands table

# Tables and Figures

Table 1: VLAN Table	3

Table 2: Connections in Branch Office	3

Table 3: Summary Table for VLAN Mapping in Branch Office	4

Table 4: Connections in Head Office	5

Table 5: Summary Table for VLAN Mapping	5

Table 6: Sub interfaces Of Router	7

Table 7: Ports Configured on Multilayered Switch 0	7

Table 8: Ports and Functions	9

Table 9: Verification commands table	13

Figure 1: Branch Office Configuration	4

Figure 2: Head Office Configuration	6

Figure 3: Router-on-a-stick has been configured	6

Figure 4: L3 Switch connecting to Router has been configured	7

Figure 5: EIGRP Routing Protocol has been configured on both router and L3 Switch	8

Figure 6: Configuring Static Route between ISP and Router(HO)	8

Figure 7: Accessing only web services through guest network	9

Figure 8: Guest-network cannot access any other device on the network	10

Figure 9: pinging HR-host to Account Server	11

Figure 10: Destination host unreachable from other devices on HR-Network	11

Figure 11: Pinging Account server from device in Account network	12

Figure 12: Pinging Account server from other networks	12

Figure 13: Ping is successful from IT Network to DNS Server	13

# References

Fibermall. (n.d.). What is an aggregation switch and its function? Fibermall. https://www.fibermall.com/blog/aggregation-switch.htm#:~:text=The%20aggregation%20switch%20is%20located,(the%20core%20layer%20switch).

Pomerium. (n.d.). East-west traffic. Pomerium.https://www.pomerium.com/glossary/east-west-traffic#:~:text=East%2Dwest%20traffic%20refers%20to,north%2Dsouth%22%20traffic).

Lecture Slides from Week 11: EIGRP & Basics of VLAN

Lecture Slides from Week 12: Inter-vlan Routing

Lecture Slides from Week 13: Introduction to ACLs